PROJECT SYNOPSIS

AquaSync

A Secure, AI-Driven Smart Irrigation Control System

| Field | Details |
| --- | --- |
| Project Title | AquaSync — Smart Irrigation System with Zero-Vulnerability IoT Architecture |
| Domain | Precision Agriculture · IoT Security · Artificial Intelligence |
| Team | Aryan Deshwal, Priya Mehta, Vikram Singh, Amara Osei |
| Institution | IIT Delhi / IARI / CDAC / NIT Kurukshetra (Collaborative) |
| Pilot Site | Hisar District, Haryana, India (47.3 ha, 3 farms) |
| Duration | October 2025 – March 2026 (6-month field evaluation) |
| Version | AquaSync v2.4 — April 2026 |
| Technology Stack | ESP32-S3 · Node.js · PostgreSQL · MQTT/TLS · React · LSTM AI |

# 1. Project Overview

AquaSync is a fully integrated smart irrigation management platform that addresses two critical and interconnected challenges facing modern agriculture: the widespread inefficiency of water use in irrigated farming, and the severe cybersecurity vulnerabilities present in virtually all existing IoT-based irrigation systems.

Agriculture consumes approximately 69% of global freshwater, yet conventional irrigation routinely wastes over 60% of applied water through imprecise scheduling, absence of soil feedback, and failure to account for weather events. At the same time, the rapid adoption of IoT sensors and actuators in farming has created a largely unsecured digital attack surface: our assessment of twelve published and commercial smart irrigation systems found that 100% lacked replay-attack protection, 91.7% had no device authentication, and 83.3% transmitted sensor data without encryption.

AquaSync resolves both problems simultaneously. The system deploys a network of ESP32-S3 sensor nodes communicating via TLS-secured, Ed25519-signed MQTT payloads to a hardened backend. An AI scheduling engine — combining the FAO-56 Penman-Monteith evapotranspiration model with an LSTM neural network — delivers irrigation schedules that adapt in real time to soil conditions, crop growth stage, and weather forecasts.

# 2. Problem Statement

2.1 Water Inefficiency

- Global agriculture withdraws ~3,900 km³ of freshwater annually; India alone accounts for ~761 km³ (89% of freshwater use)

- Traditional flood and unmanaged drip irrigation achieve only 35–65% application efficiency

- Indian farmers lose an estimated ₹4.2 trillion annually to water waste, over-irrigation crop losses, and groundwater depletion

- Climate change is reducing predictable rainfall patterns, making precision irrigation critical for food security

2.2 Cybersecurity Vacuum

- 91.7% of reviewed systems: no device authentication — any endpoint can inject false sensor data

- 100% of reviewed systems: valve commands vulnerable to replay attacks — attackers can flood or dry fields remotely

- 83.3% of reviewed systems: MQTT traffic unencrypted — farm operational data interceptable by competitors

- 100% of reviewed systems: firmware updates unsigned — trivial malware injection into field devices

- No existing open-source smart irrigation platform satisfies OWASP IoT Top Ten or ETSI EN 303 645 requirements

# 3. Objectives

- Design and implement a complete IoT sensor network for soil moisture, temperature, flow rate, pressure, pH, and rainfall monitoring

- Build a security-hardened backend API with JWT authentication, RBAC, rate limiting, anti-replay protection, and AES-256-GCM data encryption

- Develop an AI-driven irrigation scheduler integrating the FAO-56 ET₀ model with an LSTM neural network for precise, adaptive water management

- Secure the firmware supply chain with Ed25519-signed OTA updates and ESP32-S3 Secure Boot V2

- Achieve ≥35% reduction in water consumption compared to conventional irrigation on pilot farms

- Achieve zero successful security incidents during a 6-month production deployment

- Deliver a real-time dashboard with WebSocket-driven zone monitoring, alert management, and usage analytics

# 4. System Architecture

4.1 Five-Layer Architecture

| Layer | Components | Key Technologies |
| --- | --- | --- |
| Sensor Layer | ESP32-S3 nodes, soil moisture, temp, flow, pH, rain, pressure sensors | ESP-IDF, DHT22, SEN0193, YF-S201, MPX5700 |
| Communication Layer | TLS-MQTT broker, LoRaWAN gateway, Ed25519 signing | Mosquitto, TLS 1.3, mbed TLS, LoRa SX1276 |
| Backend Layer | REST API, database, auth, encryption, audit | Node.js, PostgreSQL, JWT, AES-256-GCM, Helmet |
| AI/Analytics Layer | ET₀ model, LSTM scheduler, anomaly detection | Python, TensorFlow, FAO-56, IMD Weather API |
| Dashboard Layer | Real-time web app, mobile app, alerts, reports | React, WebSocket, Chart.js, PWA |

4.2 Security Architecture: 10-Layer Defense-in-Depth

| # | Security Control | Vulnerability Addressed |
| --- | --- | --- |
| 1 | JWT + Role-Based Access Control (RBAC) | Unauthenticated API access |
| 2 | TLS 1.3 for all MQTT, HTTP, WebSocket | Plain-text interception |
| 3 | Ed25519 device signatures on every reading | Sensor data forgery |
| 4 | Nonce + timestamp anti-replay mechanism | Command replay attacks |
| 5 | AES-256-GCM encrypted data at rest | Database exfiltration |
| 6 | Rate limiting (10 auth / 300 global per 15 min) | DoS / brute-force attacks |
| 7 | Joi schema validation on all endpoints | SQL injection / buffer overflow |
| 8 | Ed25519-signed OTA + ESP32 Secure Boot V2 | Unsigned firmware injection |
| 9 | NVS secure partition for private key storage | Hardcoded credentials |
| 10 | Full immutable audit log (user/time/zone/action) | Undetected unauthorized access |

# 5. Sensor Network Specifications

| Sensor | Model | Parameter | Range | Accuracy | Sample Rate |
| --- | --- | --- | --- | --- | --- |
| Soil Moisture | SEN0193 | VWC % | 0–100% | ±2% | 5 min |
| Temperature/RH | DHT22 | °C / %RH | -40–80°C | ±0.5°C | 5 min |
| Flow Meter | YF-S201 | L/min | 1–30 L/min | ±3% | 1 sec |
| Pressure | MPX5700AP | kPa | 0–700 kPa | ±2.5% | 10 sec |
| Rain Gauge | RG11 | mm/hr | 0–300 mm/hr | ±2% | Event |
| Soil pH | E-201C | pH | 0–14 pH | ±0.1 | 15 min |

Each node runs on an 18650 LiPo cell with integrated solar charging. The ESP32-S3 enters deep sleep between readings, achieving 7 µA standby current and projected battery life exceeding 18 months at 5-minute sampling intervals. Nodes communicate via WiFi (short range) or LoRaWAN SX1276 (up to 10 km rural coverage), with automatic failover between protocols.

# 6. AI Scheduling Engine

6.1 Evapotranspiration Modeling

The primary crop water demand estimate is computed using the FAO-56 Penman-Monteith equation, the internationally recognized standard for reference evapotranspiration (ET₀). Inputs are sourced from the India Meteorological Department (IMD) API (48-hour forecast) and local weather station readings. Crop-specific water demand is computed by multiplying ET₀ by a crop coefficient (Kc) calibrated to the current growth stage, automatically updated from a built-in FAO-56 crop calendar database.

6.2 LSTM Neural Network

A two-layer Long Short-Term Memory (LSTM) network (128 hidden units per layer, trained on 24 months of site-specific data) refines the ET₀ estimate by learning systematic biases and microclimate effects not captured by the physics model. The model predicts irrigation demand for the following 24-hour period at 30-minute granularity, achieving a mean absolute error of 1.4% VWC on the held-out test set, outperforming pure ET₀-based scheduling by 23%.

6.3 Additional AI Capabilities

- Rain bypass: integrates weather forecast; automatically skips or delays irrigation when precipitation probability > 70%

- Leak detection: Isolation Forest algorithm identifies abnormal flow patterns (leaks, broken valves) in < 2 seconds

- Soil saturation guard: prevents irrigation when soil moisture exceeds field capacity to avoid root disease

- Adaptive crop coefficients: Kc values update automatically as growth stage progresses based on days-after-sowing calendar

# 7. Results — Pilot Deployment (6 Months)

| Metric | Baseline | AquaSync | Improvement |
| --- | --- | --- | --- |
| Water consumption (avg, m³/ha) | 6,163 | 3,682 | ↓ 40.3% |
| Crop yield (normalized) | 100% | 126.8% | ↑ 26.8% |
| Irrigation cost per hectare (₹) | 18,400 | 11,020 | ↓ 40.1% |
| Undetected leaks (count) | 7 | 0 | 100% detected |
| Security incidents | N/A (no monitoring) | 0 incidents | Zero-incident |
| System uptime | N/A | 99.2% | High availability |
| Alert-to-notification latency | N/A | < 2 seconds | Real-time |
| Annual net saving per farm (₹) | — | ~₹34,000 | Positive ROI |

An independent penetration test conducted in November 2025 confirmed zero successful exploits across all eight identified vulnerability categories. All replay attack attempts, SQL injection payloads, rogue firmware uploads, and unauthorized MQTT connections were detected and blocked by the respective security controls.

# 8. Technology Stack

| Component | Technology | Purpose |
| --- | --- | --- |
| Microcontroller | ESP32-S3 (Xtensa LX7, 240 MHz) | Sensor acquisition, TLS-MQTT, Ed25519 signing |
| Firmware Framework | ESP-IDF 5.2 + Arduino Core | Sensor drivers, secure boot, OTA, deep sleep |
| Communication | MQTT over TLS 1.3 / LoRaWAN | Secure bidirectional sensor-cloud messaging |
| Backend Runtime | Node.js 22 LTS + Express 4 | REST API, WebSocket server, business logic |
| Database | PostgreSQL 16 | Structured sensor data, audit logs, user management |
| Authentication | JWT (HS256) + bcrypt | Stateless auth, role-based access control |
| Encryption at Rest | AES-256-GCM | Sensor data, credentials, PII protection |
| AI Framework | Python 3.12 + TensorFlow 2.15 | LSTM model training, ET₀ computation |
| Frontend | React 19 + WebSocket + Chart.js | Real-time dashboard, mobile PWA |
| DevOps | Docker + GitHub Actions + Nginx | CI/CD, containerized deployment, TLS termination |

# 9. Project Deliverables

- Complete ESP32-S3 sensor node hardware design (KiCad schematics + PCB layout)

- Signed ESP-IDF firmware source code with Ed25519 authentication and OTA update system

- Node.js backend API with full security suite (auth, validation, encryption, audit)

- React web dashboard with real-time WebSocket zone monitoring and alert management

- React Native mobile application (iOS + Android) for field technician access

- TensorFlow LSTM model with training pipeline and 24-month benchmark dataset

- PostgreSQL database schema with migration scripts and seed data

- Docker Compose production deployment configuration with Nginx and Certbot TLS

- Complete technical documentation: architecture guide, API reference, deployment manual

- Peer-reviewed research paper submitted to IEEE Sensors Journal

- Six-month field evaluation report with raw data and statistical analysis

# 10. Future Scope

- Satellite-based soil moisture estimation integration (Sentinel-1 SAR) for farms without dense sensor coverage

- Blockchain-based water credit trading between farms using verified AquaSync consumption data

- Federated learning across farms to improve LSTM model accuracy without sharing raw data

- Integration with government Pradhan Mantri Krishi Sinchai Yojana (PMKSY) data portal

- Edge AI deployment on ESP32-S3 for autonomous scheduling during internet outages

- Multi-crop segmentation via drone imagery to apply zone-specific crop coefficients

- SaaS platform offering subscription-based access to AquaSync for smallholder farmers

AquaSync v2.4  ·  April 2026  ·  IIT Delhi / IARI / CDAC / NIT Kurukshetra

research@aquasync.io  ·  github.com/aquasync-iot  ·  aquasync.io